Notes about the sound drivers

1) Gravis UltraSound

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s4 for Gravis UltraSound). This is equivalent to the original

internal driver that came with previous versions of Impulse Tracker.

The second driver is specified by providing the correct IRQ for the

GF1 chip. (The second-to-last number of your ULTRASND environment

variable). This is an IRQ driven routine, which means that it'll

work in the background of Windows '95. But note that the timing for

this is NOT as accurate as the timing in the first driver. There is

also a possibility that multitasking OSs can sometimes (although

rarely) cause some settings to the GUS to be missed (which will cause

a note to play unexpectedly). This can be fixed just by restarting

playback. There is NO check for the correctness of the IRQ provided.

The Gravis UltraSound *CANNOT* cope with 16-bit samples greater than

256k-bytes. This is equivalent to 128k-samples. Also, 16-bit samples

cannot cross 256k boundaries on the GUS, meaning that the amount of

memory you have on the card may decrease by more than you expect when

you load a 16-bit sample.

You cannot choose the mixing rate for the GUS - the mixing rate is

dependent on the number of channels you initialise the program with

(using /Lxx)

2) InterWave - Hardware mixing ONLY. You *NEED* to have RAM onboard to use

this.

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s6 for AMD Interwave IC). This is similar to the original internal

GUS driver that came with previous versions of Impulse Tracker.

The second driver is specified by providing the correct IRQ for the

Interwave chip. (The second-to-last number of your INTERWAVE

environment variable). This is an IRQ driven routine, which means

that it'll work in the background of Windows '95. But note that the

timing for this is NOT as accurate as the timing in the first

driver. There is also a possibility that multitasking OSs can sometimes

(although rarely) cause some settings to the GUS to be missed (which

will cause a note to play unexpectedly). This can be fixed just by

restarting playback. There is NO check for the correctness of the IRQ

provided.

If you are running a multitasking OS, you MAY need to run IWINIT before

running Impulse Tracker to have more memory detected.

The Interwave driver contains handlers for two different memory modes

on the Interwave - the more memory efficient mode is where the amount

of ram is directly compatible with the interwave, the second is where

the DRAM configuration is NOT directly compatible with the interwave

and the driver has to handle the RAM slightly more explicitly, which

causes the loss of memory-usage efficiency.

Here are the modes directly compatible with the interwave:

Bank 0         Bank 1          Bank 2          Bank 3          Total

256Kb          0               0               0              256Kb

256Kb          256Kb           0               0              512Kb

256Kb          256Kb           256Kb           256Kb           1MB

256Kb          1MB             0               0              1.25MB

256Kb          1MB             1MB             1MB            3.25MB

256Kb          256Kb           1MB             0              1.5MB

256Kb          256Kb           1MB             1MB            2.5MB

1MB            0               0               0               1MB

1MB            1MB             0               0               2MB

1MB            1MB             1MB             1MB             4MB

4MB            0               0               0               4MB

* 4MB            4MB             0               0               8MB

* 4MB            4MB             4MB             4MB            16MB

* These modes cannot be handled by the first driver, so are actually

handled in the second mode.

The mixing rate for the Interwave driver is fixed at 44100Hz

(CD quality)

Bug warning: If the sound does NOT play properly, you may need to

run IWINIT before running Impulse Tracker